FLOOR MOUNTED TOILET PARTITION SPECIFICATIONS

(EMBOSSED STAINLESS STEEL – STANDARD)

SECTION 10 21 13.13

PART 1 	GENERAL

1.01		SUMMARY

A.  		Section Includes

Furnish, deliver and install all Toilet Partitions as indicated on the drawings and as required by actual conditions at the building.  The Toilet Partitions shall include the furnishing of all necessary screws, special screws, bolts, special bolts, expansion shields and all other devices necessary for the proper installation and application of the Toilet Partitions.

B.  		Related Sections

Section: 10500  Lockers

Section: 10800  Washroom Accessories

1.02		REFERENCES

A.  		Standard

All Toilet Partitions must be scheduled, supplied and installed in accordance with: Local Building Code, CGSB (Canadian Government Specifications Board), CSA (Canadian Standards Association), ANSI (American National Standards Institute), ADA (Americans with Disabilities Act).  In all cases the above references shall be taken to mean the latest edition of that particular standard including all revisions.

1.03 		SUBMITTALS

A. 		General Requirements

Make all submittals in accordance with Section: 01300

B. 		Schedules

1. Submit (4) copies of detailed shop drawings for the Consultant's/Owner's review within (2) weeks of being awarded this subcontract.

C.  		Product Data

1. Submit (2) copies of product sheets and/or catalogue cuts, of all products listed in the shop drawings.

D.  		Samples

1. Upon request, a returnable sample of the Toilet Partitions shall be submitted to the Consultant/Owner for approval not later than (10) days after requested.  All samples must be properly identified including: name of supplier, and name of manufacturer.

E.. 		Operations and Maintenance Data

1. At completion of the job, furnish to the owner (2) copies of an Owners Operation and Maintenance Manual.  The Manual shall consist of a hard cover three ring binder with the project name in the front.  Include in the manual the following information: Maintenance instructions, Catalogue pages for each product, Name/Address and phone number of the Manufacturer and their Sales Agent, Copy of the final shop drawings.

1.04 		QUALITY ASSURANCE

A.  		Substitutions

1. Manufacturers and model number listed are to establish a standard of quality.  Similar items by approved manufacturers that are equal in design, function, quality and finish may be accepted upon prior written approval from the Architect/Owner.

2. All requests for acceptable substitutions must be made in writing and submitted to the Architect at least 14 days prior to tender closing.  If requested, all requests for substitutions must be accompanied by product literature and actual product samples.

B.  		Supplier Qualifications

1. Toilet Partition shop drawings and Toilet Partitions shall be procured from a source of supply approved by the Consultant/Owner/Architect.  Supplier is responsible for the complete Toilet Partition subcontract.

1.05 		DELIVERY, STORAGE AND HANDLING

A.  		Marking and Packaging

1. Toilet Partitions must be delivered to the job site in the manufacturers' original packages and marked to correspond with the approved shop drawings.

B. 		 Delivery

1. Toilet Partitions must be delivered in an amount of time deemed appropriate by the Consultant/Owner.

1.06 		WARRANTY

A.  		Written Guarantee

1. The Toilet Partition manufacturer shall guarantee all Toilet Partitions by written certification, for a period of (5) years from date of receipt by customer, against any defects in design, materials and workmanship.

1.07 		MAINTENANCE

A.  		Maintenance

1. Upon request, at completion of the project, the Toilet Partition supplier may be required to brief Owner's maintenance staff regarding proper care of Toilet Partitions, such as: required lubrications, adjustments, cleaning, etc.

PART 2 	PRODUCTS

2.01 		MANUFACTURERS

A.  		Approved Manufacturers

Only those manufacturers names and product numbers listed herein, are approved for use on this project.  All other manufacturers must request approval as per section (1.04 - A - Substitutions).  Absolutely no variations from listed and preapproved items will be permitted.

Approved manufacturer(s):

1. Hadrian Manufacturing Inc.

2.02 		MATERIALS

A. 	Construction: Doors, Panels and Pilasters shall be constructed of two sheets of type 304, Embossed stainless steel with 5WL® pattern, laminated under pressure to a “verticel” (1/2”) honeycomb core for impact resistance, rigidity and sound deadening. Formed edges to be welded together and interlocked, under tension, with a roll-formed oval crown stainless steel locking bar, mitred, welded and ground smooth at the corners. Honeycomb to be of virgin, long fiber paper with a maximum 12.5mm (1/2”) cell size.

B.  	Doors: Shall be 25mm (1") thick with cover sheets not less than 22-gauge (0.8mm).

C.	Panels: Shall be 25mm (1") thick with cover sheets not less than 22-gauge (0.8mm).

E. 	Pilasters: Shall be 32mm (1.25") thick with cover sheets not less than 18-gauge (1.2mm).

F.	Hardware and Fittings: All panel and pilaster brackets and all door hardware shall be chrome plated zinc die castings, standard. Fasteners are 12 x 1-3/4 and 12 x 5/8 TR-27 6-lobe security screws. Doors shall be equipped with a gravity type hinge mounted on the lower pilaster hinge bracket. Doors shall be equipped with a gravity type hinge mounted on the lower pilaster hinge bracket. Door hinges shall be the wraparound type and adjustable to permit the door to come to rest at any position when not latched. Each door to be fitted with a combined coat hook and bumper and a concealed latch, with face mortised flush with edge strip of door Barrier-free doors shall include thumbturn lever to activate latch without fingertip grip application. Both standard and barrier-free latches shall have a turn slot designed to allow emergency access from exterior. The combined stop and keeper shall have a 19mm (0.75") diameter bumper locked in place. Threaded upper hinge pin shall have a metal core and self-lubricating nylon sleeve to ensure smooth, quiet operation. Pilaster shoes shall be a welded one-piece design made from polished stainless steel. Two-piece shoes that can disassemble when kicked are unacceptable.

Stainless Steel Hardware Option:

- All pilaster, panel and screen brackets are stamped stainless steel, #4 brushed finish.

- All door wrap around hinge brackets are cast stainless steel, #4 brushed finish.

- All 12 x 1-3/4 and 12 x 5/8 TR-27 6-lobe security screws are stainless steel, #4 brushed finish.

- All pilaster shoes are stainless steel, #4 brushed finish.

- All inner, barrier free inner, outer, door pulls, stop and keepers, coat hooks and door castings are standard zinc die castings, #4 brushed finish.

- All other components not outlined here are standard Hadrian hardware items.

2.03 		FINISH

A.  		Type 304, Embossed stainless steel with 5WL pattern.

PART 3  	EXECUTION

3.01 		EXAMINATION

A.  		Site Preparation

1. The contractor must examine all site conditions that would prevent the proper application and installation of Toilet Partitions.  Any defect must be immediately identified and corrected, prior to the installation of the Toilet Partitions.

3.02 		INSTALLATION

A. 		Mounting Locations

1. All Toilet Partitions must be mounted according Manufacturers standard locations and those specified on the drawings.

3.03 		FIELD QUALITY CONTROL

A.  		Inspection

1. After installation has been completed, provide for a site inspection of all Toilet Partitions to determine that all items have been supplied and installed as per the enclosed details.  Also, check the operation and adjustment of all Toilet Partitions.  Any discrepancies, or malfunctioning product, must be reported to the Architect immediately.

3.04 		ADJUSTMENT AND CLEANING

A.  		Final Preparation

1. At final completion, Toilet Partitions shall be left clean and free from disfigurement.  Make all final adjustments.  Where Toilet Partitions are found defective, repair or replace or otherwise correct as directed.

3.05 		PROTECTION

A.  		Site Protection

1. The Contractor must provide for the proper protection of all Toilet Partitions until the owner accepts the project as complete.

3.06 		TOILET PARTITION SCHEDULE

A.  		Schedule

1. Provide Toilet Partitions as specified in all above sections and as per the detailed Architectural Drawings.

Hadrian Manufacturing Inc., 965 Syscon Road, Burlington, ON L7L 5S3 Tel: 905-333-0300 Fax: 905-333-1841

Hadrian Inc., 7420 Clover Avenue, Mentor, OH 44060 Tel: 440-942-9118 Fax: 440-942-9618

www.hadrian-inc.com